# Cocktail Mixing System Documentation

## Table of Contents

- System Overview

- Prerequisites

- Installation

- Raspberry Pi Setup

- Windows Setup

- Application Usage

- Troubleshooting

- Maintenance

## System Overview

The Cocktail Mixing System is an automated cocktail preparation system that combines:

- A web-based user interface for cocktail selection and management

- A Python backend for processing orders and controlling hardware

- Serial communication with Arduino for pump control

- Firebase integration for data synchronization

- Support for both Windows and Raspberry Pi platforms

Main Interface The main interface of the Cocktail Mixing System, showing the cocktail selection menu and system status.

## Prerequisites

### Common Requirements

- Python 3.11 or later

- Node.js and npm

- Git

- Firebase account and credentials

- Arduino with pump control setup

### Raspberry Pi Specific

- Raspberry Pi 4 (recommended) or Raspberry Pi 3

- MicroSD card (16GB or larger)

- Power supply

- HDMI cable and monitor (for initial setup)

- Keyboard and mouse (for initial setup)

- Internet connection

## Installation

The installation process is automated using the installer.sh script, which handles all the necessary setup steps. Simply run:

# Run the installer
./installer.sh

The installer script will:

- Install all required Python dependencies

- Set up Node.js and Electron

- Configure Firebase

- Set up the application for your platform (Windows or Raspberry Pi)

- Configure auto-start settings

For manual installation or troubleshooting, refer to the detailed steps in the troubleshooting section.

## Raspberry Pi Setup

### 1. Initial Setup

# Update system
sudo apt update
sudo apt upgrade

# Install required packages
sudo apt install python3-pip python3-venv git nodejs npm tmux cron

### 2. Configure Serial Port

sudo raspi-config

- Navigate to "Interface Options"

- Enable Serial Port

- Disable Serial Console

- Reboot when prompted

### 3. Install Application

# Clone repository
git clone https://github.com/DeamonKing/new-repo.git /opt/cocktail-mixer
cd /opt/cocktail-mixer

# Install dependencies
pip3 install -r requirements.txt
npm install -g electron

### 4. Configure Auto-Start

The installer will automatically set up crontab for startup. To verify:

crontab -l

Should show:

@reboot /opt/cocktail-mixer/start_at_boot.sh

## Windows Setup

### 1. Install Required Software

- Install Python 3.11 from python.org

- Install Node.js from nodejs.org

- Install Git from git-scm.com

### 2. Install Application

# Clone repository
git clone https://github.com/DeamonKing/new-repo.git
cd new-repo

# Install dependencies
pip install -r requirements.txt
npm install -g electron

### 3. Configure Auto-Start

- Create a shortcut to start_at_boot.sh

- Place the shortcut in the Startup folder:

  - Press Win+R

  - Type shell:startup

  - Copy the shortcut to this folder

## Application Usage

### Managing Cocktails

Cocktail Management The cocktail management screen where users can add, edit, and delete cocktails from the system.

- Add New Cocktail

  - Click "Add Cocktail"

  - Enter cocktail details

  - Assign ingredients to cocktail

  - Set drink intensity

  - Save the cocktail

### Managing Ingredients

Ingredient Configuration The ingredient configuration interface for managing available ingredients and their pump assignments.

- Add New Ingredient

  - Click "Add Ingredient"

  - Enter ingredient details

  - Assign to a pump

  - Save the ingredient

- Update Ingredients remarks

  - Select ingredients to modify

  - Update quantities or pump assignments

  - Save changes

### Making Cocktails

Drink Intensity The drink intensity selection screen where users can choose between light, medium, and strong drink options.

- Select a cocktail from the menu

- Choose drink intensity (light/medium/strong)

- Click "Make Cocktail"

- The system will automatically:

  - Control pumps for precise measurements

  - Monitor the mixing process

  - Display progress

  - Notify when complete

Mixing Process The mixing process screen showing real-time progress of cocktail preparation.

### Update APP

- Check for newer versions of the application.

- Download and install the latest version with a simple click of the "Update" button.

- Ensure the system is running the most up-to-date software.

## Configuration and Credential Files

- jecon-cocktail-machine-firebase-adminsdk-fbsvc-0fecbeb26b.json - Firebase Admin SDK credentials file for authentication

- firebase_config.py - Contains Firebase configuration settings and initialization

- firebase_credentials_encoded.txt - Encoded Firebase credentials for secure storage

- config.py - General application configuration settings

- requirements.txt - Python package dependencies list

- package.json - Node.js package dependencies and scripts

## Main Application Files

- app.py - Main application file containing the core functionality

- image_handler.py - Handles image processing and management

- loading_screen.py - Manages the loading screen functionality

## Git and Version Control

- .git/ - Git repository directory

- .gitignore - Specifies which files Git should ignore

- git-pull.py - Script for pulling updates from Git repository

- git-push.py - Script for pushing changes to Git repository

## System and Startup Scripts

- run.sh - Main script to run the application

- start_at_boot.sh - Script to start the application at system boot

- keyboardstart.sh - Script to start keyboard-related functionality

- keyboardstop.sh - Script to stop keyboard-related functionality

## Utility Scripts

- encode_credentials.py - Script for encoding credentials securely

## Directories

- static/ - Contains static files like images, CSS, and JavaScript

- node_modules/ - Contains Node.js dependencies

- __pycache__/ - Python bytecode cache directory

- backups/ - Directory for storing backup files

- .vscode/ - Visual Studio Code configuration directory

- Jecon_CM_installer/ - Directory containing installer files

## Static Directory Contents

### Frontend Files

- index.html - Main HTML file for the web interface

- style.css - CSS styles for the application

- script.js - Main JavaScript file for frontend functionality

- colorPicker.js - JavaScript for color picker functionality

- main.js - Main JavaScript entry point

- preload.js - Preload script for Electron app

### Data Files

- config.json - Configuration data for the application

- products.json - Product information and data

- db.json - Database file containing application data

### Processing Files

- processing - File indicating processing status

- processing_complete - File indicating processing completion

### Images Directory

- img/ - Directory containing image assets

## Project Overview

This is a cocktail machine application with both backend (Python) and frontend (JavaScript/HTML/CSS) components. It uses Firebase for backend services and data storage. The application includes features for managing products, handling images, and providing a user interface for cocktail machine operations.

## Troubleshooting

## Common Issues

- Application Won't Start

  - Check if port 5000 is available

  - Verify Python and Node.js installations

  - Check system logs

- Serial Communication Problems

  - Verify Arduino connection

  - Check serial port configuration

  - Review system logs

- Firebase Sync Issues

  - Verify internet connection

  - Check Firebase credentials

  - Review error logs

- Pump Control Problems

  - Check serial connection

  - Verify pump assignments

  - Review error logs

## Support

For additional support:

- Check the GitHub repository

- Review the troubleshooting section

- Contact system administrator